import numpy as np

import random

import matplotlib.pyplot as plt

# -------------------------

# Environment Settings

# -------------------------

GRID_SIZE = 12

NUM_OBSTACLES = 6

START = (0, 0)

GOAL = (11, 11)

ACTIONS = ["UP", "DOWN", "LEFT", "RIGHT"]

TOTAL_EPISODES = 1000

MAX_STEPS = 150

# Load trained Q-table

Q = np.load("q_table_v3.npy")

# APF Parameters

K_ATTRACTIVE = 1.0

K_REPULSIVE = 4.0

REPULSIVE_RANGE = 3

# Local minima escape parameters

STUCK_THRESHOLD = 5

USE_RANDOM_KICK = True

USE_VIRTUAL_OBSTACLE = True

VIRTUAL_OBS_DURATION = 4

# -------------------------

# Helper Functions

# -------------------------

def state_to_index(state):

return state[0] * GRID_SIZE + state[1]

def move(position, action):

x, y = position

if action == 0:  # UP

x = max(x - 1, 0)

elif action == 1:  # DOWN

x = min(x + 1, GRID_SIZE - 1)

elif action == 2:  # LEFT

y = max(y - 1, 0)

elif action == 3:  # RIGHT

y = min(y + 1, GRID_SIZE - 1)

return (x, y)

def manhattan_distance(a, b):

return abs(a[0] - b[0]) + abs(a[1] - b[1])

def euclidean_distance(a, b):

return ((a[0] - b[0]) ** 2 + (a[1] - b[1]) ** 2) ** 0.5

def random_obstacles():

obstacles = []

while len(obstacles) < NUM_OBSTACLES:

pos = (

random.randint(0, GRID_SIZE - 1),

random.randint(0, GRID_SIZE - 1)

)

if pos != START and pos != GOAL and pos not in [o['pos'] for o in obstacles]:

speed = random.choice([1, 2])

target = (

random.randint(0, GRID_SIZE - 1),

random.randint(0, GRID_SIZE - 1)

)

obstacles.append({

'pos': pos,

'speed': speed,

'target': target

})

return obstacles

def move_toward_target(pos, target):

x, y = pos

tx, ty = target

if x < tx:

x += 1

elif x > tx:

x -= 1

elif y < ty:

y += 1

elif y > ty:

y -= 1

return (x, y)

def move_obstacles(obstacles):

for obs in obstacles:

for _ in range(obs['speed']):

if random.random() < 0.7:

new_pos = move_toward_target(obs['pos'], obs['target'])

else:

action = random.randint(0, 3)

new_pos = move(obs['pos'], action)

if new_pos == GOAL:

continue

obs['pos'] = new_pos

if manhattan_distance(obs['pos'], obs['target']) == 0:

obs['target'] = (

random.randint(0, GRID_SIZE - 1),

random.randint(0, GRID_SIZE - 1)

)

return obstacles

def obstacle_positions(obstacles):

return [o['pos'] for o in obstacles]

def safe_action(robot, action, obstacles):

next_pos = move(robot, action)

return next_pos not in obstacle_positions(obstacles)

# -------------------------

# Q-Learning Policy

# -------------------------

def q_learning_action(robot, obstacles):

state_idx = state_to_index(robot)

sorted_actions = np.argsort(Q[state_idx])[::-1]

for a in sorted_actions:

if safe_action(robot, a, obstacles):

return a

return random.randint(0, 3)

# -------------------------

# APF Policy (Corrected)

# -------------------------

def attractive_potential(position):

# Calculate scalar potential energy: U_att = 0.5 * k * distance^2

dist = euclidean_distance(position, GOAL)

return 0.5 * K_ATTRACTIVE * (dist ** 2)

def repulsive_potential(position, obstacles):

# Calculate scalar potential energy: U_rep = 0.5 * k * (1/d - 1/d0)^2

u_rep_total = 0

for obs in obstacles:

obs_pos = obs['pos']

dist = euclidean_distance(position, obs_pos)

if dist == 0:

dist = 0.1  # Prevent division by zero

if dist <= REPULSIVE_RANGE:

strength = 0.5 * K_REPULSIVE * ((1 / dist) - (1 / REPULSIVE_RANGE)) ** 2

u_rep_total += strength

return u_rep_total

def apf_action(robot, obstacles, recent_positions, virtual_obstacle=None):

best_action = None

best_potential = float('inf') # We want to MINIMIZE potential

# Detect local minima / stuck behavior

stuck = False

if len(recent_positions) >= STUCK_THRESHOLD:

last_positions = recent_positions[-STUCK_THRESHOLD:]

if len(set(last_positions)) <= 2:

stuck = True

# Random kick strategy

if stuck and USE_RANDOM_KICK:

valid_actions = []

for a in range(4):

if safe_action(robot, a, obstacles):

valid_actions.append(a)

if valid_actions:

return random.choice(valid_actions), robot, VIRTUAL_OBS_DURATION

# Virtual obstacle strategy

effective_obstacles = list(obstacles)

if virtual_obstacle is not None and USE_VIRTUAL_OBSTACLE:

effective_obstacles.append({'pos': virtual_obstacle, 'speed': 0, 'target': virtual_obstacle})

if stuck and USE_VIRTUAL_OBSTACLE and virtual_obstacle is None:

virtual_obstacle = robot

# Evaluate each possible next action

for action in range(4):

if not safe_action(robot, action, effective_obstacles):

continue

next_pos = move(robot, action)

# Calculate scalar potential energy at the next position

u_att = attractive_potential(next_pos)

u_rep = repulsive_potential(next_pos, effective_obstacles)

total_u = u_att + u_rep

# We want the cell with the LOWEST potential energy

if total_u < best_potential:

best_potential = total_u

best_action = action

if best_action is None:

return random.randint(0, 3), virtual_obstacle, VIRTUAL_OBS_DURATION

return best_action, virtual_obstacle, VIRTUAL_OBS_DURATION if virtual_obstacle else 0

# -------------------------

# Runner

# -------------------------

def run_agent(agent_type):

success = 0

crash = 0

loop = 0

timeout = 0

steps_log = []

print(f"\n===== RUNNING {agent_type.upper()} AGENT =====\n")

for episode in range(1, TOTAL_EPISODES + 1):

robot = START

obstacles = random_obstacles()

visited_positions = []

recent_positions = []

virtual_obstacle = None

virtual_timer = 0

result = 'TIMEOUT'

for step in range(1, MAX_STEPS + 1):

if agent_type == 'q_learning':

action = q_learning_action(robot, obstacles)

else:

action, virtual_obstacle, virtual_timer = apf_action(

robot,

obstacles,

recent_positions,

virtual_obstacle

)

new_robot = move(robot, action)

obstacles = move_obstacles(obstacles)

obs_positions = obstacle_positions(obstacles)

if new_robot in obs_positions:

crash += 1

result = 'CRASHED'

steps_log.append(step)

break

if new_robot == GOAL:

success += 1

result = 'REACHED GOAL'

steps_log.append(step)

break

recent_positions.append(new_robot)

if virtual_timer > 0:

virtual_timer -= 1

else:

virtual_obstacle = None

visited_positions.append(new_robot)

if visited_positions.count(new_robot) > 4:

loop += 1

result = 'LOOP'

steps_log.append(step)

break

robot = new_robot

if step == MAX_STEPS:

timeout += 1

steps_log.append(step)

print(f"Episode {episode:03d} | {result:<13} | Steps: {steps_log[-1]}")

return {

'success': success,

'crash': crash,

'loop': loop,

'timeout': timeout,

'steps': steps_log

}

# -------------------------

# Run Both Agents

# -------------------------

q_results = run_agent('q_learning')

apf_results = run_agent('apf')

# -------------------------

# Final Comparison

# -------------------------

print("\n================ FINAL COMPARISON ================")

print("\nQ-LEARNING RESULTS")

print(q_results)

print("\nAPF RESULTS")

print(apf_results)

print("\nSuccess Rate Comparison")

print(f"Q-Learning Success Rate : {q_results['success']}%")

print(f"APF Success Rate        : {apf_results['success']}%")

# -------------------------

# Comparison Graph

# -------------------------

labels = ['Reached Goal', 'Crashed', 'Loop', 'Timeout']

q_values = [

q_results['success'],

q_results['crash'],

q_results['loop'],

q_results['timeout']

]

apf_values = [

apf_results['success'],

apf_results['crash'],

apf_results['loop'],

apf_results['timeout']

]

x = np.arange(len(labels))

width = 0.35

plt.figure(figsize=(10, 6))

plt.bar(x - width / 2, q_values, width, label='Q-Learning')

plt.bar(x + width / 2, apf_values, width, label='APF')

plt.xticks(x, labels)

plt.ylabel('Number of Episodes')

plt.title('Q-Learning vs APF Comparison')

plt.legend()

plt.grid(True)

plt.tight_layout()

plt.show()